Peer Reviewed:  								Charmaine Lum

December 7, 2018

Technical Documentation

Cooling System Vents/Radiator Mount:

The purpose of this tutorial is to create a 3-D printed mount made of any type of plastic material. Usually we strive for a translucent green 3-D printer filament because that kind of material is meant to sustain more temperature sensitive components of the WAM-V (which was what we used for the high current box). But due to limited resources of the translucent green 3-D printer filament, we opted to use a white one for our second radiator mount for the low current box. The material serves the same function but not as temperature sensitive as the green colored.

Materials Required:

- Translucent Green 3-D Printer Filament (or any available color you can find)

- 3-D Printer big enough to print a 16 in. mount in length

- 5/16 Fasteners, 5/16 Washers (approximately 50 EA), 5/16 Plated Nuts (7 EA)

  - Two 4 in. 5/16 fasteners and three 6 in. 5/16 fasteners

- Phillips screws (These should come with the radiator when you purchase the box)

- Steel Piping for covering of

- 13mm Wrench

- Drill with drill bit “Q” for 5/16 fasteners and ½ space needle to cut out a hole for cooling tubes

- Coils to connect the cooling tubes to the radiator

Specifications:

Schematics for the radiator mount:

Installation Process:

- Collect all your materials stated in the requirements above

- Start off by marking your dimensions on your designated box

  - In our case, we used an empty plastic box as the low current box to attach the radiator mount

  - The placement of the radiator on the low current box should be an exact mirror (opposite sides of each other) to the high current box

  - The reasoning behind this is because there is a triangular beam on the WAM-V in the middle of the two boxes

- The dimensions for where the holes should be cut are:

  - On the left side of whichever short end face, you are using of the box:

    - The 1st mark should be 1 and 14/16 inches coming from the top

    - From Mark 1, measure Mark 2 up to 2.5 inches

    - From Mark 2, measure Mark 3 up to 3 and ¾ inches

    - Then from there, measure Mark 4 up to 4 and 14/16 inches

  - In the center of the box:

    - Coming from the left side, the 5th mark should be ¾ inches horizontally and 1.5 inches vertically

      - The 7th mark should line up with the dimensions above and land on the angled part of the box

    - The 6th mark is marked: 2.85 inches vertically and 2.25 inches horizontally

- Use Drill bit “Q” for Marks 1,4,5,6,7 and drill a hole straight through

- Use space needle “1/2” for Marks 2 & 3 and drill a hole straight through

- To assemble the radiator to the mount:

  - Place the 4 in. fasteners should be at holes 1 & 4 and the 6 in. fasteners should be at holes 5,6,7

  - Then use Philips screws and screw them at the holes of the radiator (should use all 10)

- Create 3 aluminum shafts for each of the three 6 in. fasteners. Each length should be approximately 4 inches long

  - The shaft that will be covering the fastener in hole 7 should be sanded to the angle of degree of the box (roughly 45 degrees)

  - Place them onto the three 6 in. fasteners before officially attaching the entire mount onto the low current box

- Attach the entire mount to the designated holes

  - Apply a plated nut on both sides of each of the 4 in. fasteners and a washer on the inside area of the box

  - For the remaining three 6 in fasteners, apply roughly 8-10 washers on each fastener and tighten it with a plated nut

  - It should look like this:

The product should look like this: